Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Electronic Safety and Security

Common Work Results for Electronic Safety and Security

Network Video Recorders

WireLess Access Control

- – GENERAL

  - SUMMARY

    - Section Includes

      - Storage Area Network Electronic Safety and Security

      - Cloud Based Storage for Electronic Safety and Security

      - Storage Management Software for Electronic Safety and Security

      - Communications Equipment for Electronic Safety and Security

    - Related Sections

      - [Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

| STANDARD | NA |
| --- | --- |

  - SYSTEM DESCRIPTION

    - Section Includes

      - WireLess Access Control

  - SUBMITTALS

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Dimensional Drawings; include

      - Overall device dimensions.

      - Dimensions specific for installation.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - List certifying bodies (FCC,CE, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - WARRANTY

    - Provide manufacturer’s warranty covering [3] years for replacement and repair of defective equipment. Warranty varies country to country.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

ZHEJIANG DAHUA VISION TECHNOLOGY CO., LTD.

Address: No.1399, Binxing Road, Binjiang District, Hangzhou, P.R. China

Postcode: 310053

Tel: +86-571-87688888 28933188

Email: dhoverseas@dhvisiontech.com

Website: www.dahuasecurity.com

Substitutions:

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - WireLess Access Control | DH-EAC256

| Hardware | Modular Switch/Fixed Switch | Fixed |
| --- | --- | --- |
| Hardware | PoE Power Supply | No |
| Hardware | Description of Function Slots | Port 1-5：5 × 10/100/1000Base-T RJ-45 |
| Hardware | Reset Button | 1 |
| Hardware | Power Supply | Built-in power supply: 100–240 VAC, 50/60 Hz, 2 A |
| Hardware | Power Consumption | Idling: 0.96 W; Full load: 12 W |
| Hardware | Protection | IP20 |
| Function | 802.11 LAN Protocols | 802.11b; 802.11a; 802.11g; 802.11n; 802.11ac; 802.11ax |
| Function | CAPWAP | Layer 2 network topology between an AP and AC; Enable an AP to automatically discover an accessible AC; Enable an AP to automatically download configurations from an AC |
| Function | Roaming | Intra-AC roaming |
| Function | Forwarding | Local forwarding; Centralized forwarding |
| Function | Wireless QoS | AP-based bandwidth control; WLAN-based bandwidth control; User-based static and smart speed control; Fair balancing |
| Function | User Isolation | WLAN-based user isolation |
| Function | User Management | Access control based on SSID user number |
| Function | RF Management | Country code configuration; Manual configuration of transmitting power; Manual configuration of working channel; Automatically allocation of working channel; Network optimization well deployed; Scheduled reallocation of wireless channel |
| Function | WAN Port Function | Dynamic IP; Static IP; PPPoE; IP/Mask configuration; MTU configuration; TCP MSS configuration; Network bandwidth configuration; Gateway address assignment; NQA configuration to probe address |
| Function | Multi-WAN Load Balancing | Distributing network traffic equally; Ratio load balancing; Multi-link for load balancing |
| Function | Link Detection | Enable and disable link detection; ICMP probe; TCP probe |
| Hardware | Ethernet Port | 5 |
| Function | IP Routing | Static Routing; Policy-based routing |
| Function | VPN | L2TP VPN; PPTP VPN |
| Function | DHCP Function | DHCP Server; DHCP Client |
| Function | Security | Portal Authentication; PPPoE Authentication; IP/MAC Authentication; IP/MAC Binging; Intranet Anomaly Detection; Intranet Attack Protection; WAN Ping Forbid; Virtual AP; Multiple SSIDs; SSID hiding; WPA/WPA2; WPA3; WEP(WEP/WEP128); TKIP |
| Function | QoS/ACL | Traffic supervision; Congestion management; Congestion avoidance; Traffic shaping; IP-based speed limit; Traffic classification |
| Function | Equipment Management | Web Management; TFTP; FTP; NTP; System Log; Ping |
| Performance | Default Number of Manageable APs | 256 |
| Performance | Maximum Number of Manageable APs | 256 |
| Hardware | Ethernet Port Speed | 10/100/1000 Mbps |
| Performance | Maximum Number of Configurable APs | 256 |
| Performance | Users Capacity | 4K |
| Performance | Firewall | 500 Mbps |
| Performance | Forwarding Capability | 500 Mbps |
| Performance | CPU Frequency | 880 MHz |
| Performance | Processor | Dual core |
| Performance | MTBF | 40,000 hours |
| Performance | Memory | 512 MB |
| Performance | Flash | 16 MB |
| Performance | Packet Buffer Size | 2 MB |
| Performance | MAC Address List | 1024 |
| Performance | Entries of MAC Address Filtering Table | 32 |
| Performance | IPv4 Forwarding | 1 Gbps |
| Performance | Static Routes | 200 |
| Performance | Max. Number of Static DNS Records | 256 |
| Performance | Max. Number of DDNS Records | 4 |
| Performance | Static ARP | 1K |
| Performance | Dynamic ARP | 2K |
| Performance | ACL | 1K |
| Performance | Max. Number of Tunnels | 128 |
| Performance | Max. Number of Sessions | 128 |
| Performance | Max. Number of Concurrent Connections | 200K |
| Performance | New Connections | 6K |
| Performance | Max. Number of Virtual Servers | 100 |
| Performance | Max. Number of Port Triggering | 20 |
| Performance | Aging Time of Port Triggering | 10 min |
| Performance | Number of NAT | 30 |
| General | Operating Humidity | 5%–95% (RH), non-condensing |
| General | Storage Temperature | -40 ℃ to +70 ℃ (-40 ℉ to 158 ℉) |
| General | Storage Humidity | 5%–95% (RH), non-condensing |
| General | Statics Protection | Air discharge: 8 kV; Contact discharge: 6 kV |
| General | Lightning Protection | Common mode: 2 kV; Differential mode: 1 kV |
| General | Net Weight | 2.10 kg (4.63 lb) |
| General | Gross Weight | 2.58 kg (5.69 lb) |
| General | Product Dimensions | 440 mm × 221 mm × 44 mm (17.32" × 8.70" × 1.73")(L × W × H) |
| General | Packaging Dimensions | 500 mm × 297 mm × 91 mm (19.69" × 11.69" × 3.58")(L × W × H) |
| General | Installation | Desktop mount |
| Hardware | LAN Port | 4 × 10/100/1000 Mbps |
| Hardware | WAN Port | 1 × 10/100/1000 Mbps, (you can switch between the 3 LAN ports and WAN ports） |
| General | Operating Temperature | –20 °C to +55 °C (–4 °F to +131 °F) |

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION